IoT Cat Toy System: Code Overview Sheet

Project Summary: This project is an interactive IoT cat toy system powered by three ESP32s: Base, Sensor, and Trigger. The Sensor ESP uses an ultrasonic sensor to detect paw movement and a photoresistor (LDR) to check ambient lighting conditions. When the light is low and a paw is detected, it signals the Base ESP. The Base ESP then activates a DC motor to rotate the turret and a servo to move a laser pointer vertically. The Trigger ESP contains a manual override button that only activates if the photoresistor detects a low-light environment.

Main Components & GPIO Mapping:

| Component | Description | GPIO Pin(s) | ESP Role |
| --- | --- | --- | --- |
| Ultrasonic Sensor | Detects paw movement | Trig: GPIO32 | Sensor |
|  |  | Echo: GPIO14 |  |
| DC Motor + Encoder | Rotates turret base | IN1: GPIO25, IN2: GPIO26 | Base |
|  | Position feedback via encoder | ENC_A: GPIO33, ENC_B: GPIO27 | Base |
| Servo Motor | Moves laser up/down | Signal: GPIO12 | Base |
| LED | System status indicator | GPIO13 | Base |
| Photoresistor (LDR) | Senses ambient light level (voltage divider) | A0: GPIO36 | Sensor |
| Button | Manual trigger input | GPIO5 (example) | Trigger |

Mosfet STP16NF06L and Mosfet bs170

Sender Code Explanation

from machine import ADC, Pin, time_pulse_us

from time import sleep_ms

import network

import espnow

- This sets up modules for reading sensor input, setting up network capabilities, communicating with ESP-NOW protocol, and handling timing.

# ─────────── GPIO SETUP ───────────

TRIG = Pin(32, Pin.OUT, value=0)

ECHO = Pin(14, Pin.IN)

ldr = ADC(Pin(34))

- TRIG is the variable used to trigger the ultrasonic sensor

- ECHO is the variable that receives the echo signal from ultrasonic sensor

- Ldr is the analoug input pin connected to the light dependent resistor (LDR)

# ─────────── LDR SETUP ───────────

ldr.atten(ADC.ATTN_11DB)

LIGHT_THRESHOLD = 1000

- This basically sets a threshold to determine whether it’s “light” or “dark”.

# ─────────── ESP-NOW SETUP ───────────

sta = network.WLAN(network.STA_IF)

sta.active(True)

sta.disconnect()

e = espnow.ESPNow()

e.active(True)

peer = b'\x10\x06\x1c\x0c\xe1\x44' # reciever MAC Addresss

e.add_peer(peer)

- This code initializes the Wi-Fi interface that prepares the ultrasonic sensor and LDR to sense distance and light.

- Sets up ESP-NOW protocol to send data to another ESP32.

# ─────────── DISTANCE FUNCTION ───────────

def distance_cm():

TRIG.off(); sleep_ms(2)

TRIG.on();  sleep_ms(10)

TRIG.off()

try:

us = time_pulse_us(ECHO, 1, 30_000)

return round((us / 2) / 29.1, 2)

except OSError:

return -1

- Sends a 10ms trigger pulse  to the ultrasonic sensor

- Converts time to distance in centimeters.

# ─────────── DISTANCE FILTERING ───────────

def stable_distance():

readings = []

for _ in range(5):

d = distance_cm()

if 0 < d < 100:

readings.append(d)

sleep_ms(10)

if readings:

readings.sort()

return readings[len(readings) // 2]

else:

return -1

- Takes 5 distance reading and returns the median value if the readings are valid.

# ─────────── PAW DETECTION ───────────

def paw_detected():

print("Paw Detected!")

e.send(peer, "PAW")

# ─────────── WAIT FOR LIGHTS OFF ───────────

while True:

light = ldr.read()

print(f"LDR value: {light}", end='  ')

if light > LIGHT_THRESHOLD:

print("Lights are ON — turn off lights to start play.")

else:

print("Lights OFF — starting play!")

break

sleep_ms(500)

# ─────────── MAIN LOOP ───────────

while True:

# Wait for lights OFF to start play

while True:

light = ldr.read()

print(f"LDR value: {light}", end='  ')

if light > LIGHT_THRESHOLD:

print("Lights are ON — waiting...")

else:

print("Lights OFF — starting play!")

break

sleep_ms(500)

# Set new ultrasonic baseline before play begins

baseline_cm = stable_distance()

print(f"Baseline distance: {baseline_cm:.2f} cm")

while True:

# Check if lights turned back on mid-play

light = ldr.read()

if light > LIGHT_THRESHOLD:

print("Lights turned ON — stopping play.")

break  # Exit play mode

d = stable_distance()

if d == -1:

print("Sensor error or out of range")

continue

delta = baseline_cm - d

print(f"dist={d:.2f} cm  Δ={delta:.2f} cm")

if delta > 5:

confirm = 0

for _ in range(8):

dd = stable_distance()

if dd != -1 and (baseline_cm - dd > 5):

confirm += 1

sleep_ms(50)

if confirm >= 6:

paw_detected()

sleep_ms(2000)

else:

print("False alarm")

sleep_ms(200)

else:

baseline_cm = 0.9 * baseline_cm + 0.1 * d

sleep_ms(50)